# The pack contains recorded data for driver 1, and base data (the path they were following).

# If you run trackinit.m you’ll see the base path for the whole (1.5 hour) route around villages to the East of Loughborough, and the position data for driver 1 on a section between Hoby and Thrussington (in thick blue).  On my PC this takes a while to run, as the path is long.  If you look up the placenames on google maps (other mapping services are available) you should be able to follow the route (we always drove clockwise as shown on the map).

# My research used the speed and steer that drivers had when driving freely, on open B-class roads – so you’ll see none of the driver data passes through villages – it was all on derestricted sections of road, with no car in front.  In the driver1.mat file you’ll find numbered data giving the following information.  The numbers just refer sequentially to the sections of data recorded (further details in the testrecord spreadsheet).

# t (time, secs)

# X,Y, Z (position in metres – note that you need to plot this counterintuitively, as we use a right handed axis system with Z positive down and Y to the right – SAE).  Trackinit does this by plotting X on the y axis, and Y on the x axis.

# ax, ay  (long, lat accel – m/s^2)

# d, dsw  (steer angle in degrees, at the steered wheels, steering wheel)

# psi  (absolute yaw angle in degrees)

# u, v (long, lateral velocity in car axis system – m/s)